1. ESP32 (Main Controller – the brain)

ESP32 is the center of the system.

Every part is connected to ESP32.

It:

Receives data from RFID and fingerprint

Checks if the user is valid

Controls display, LED and buzzer

Sends data to cloud

So:

All sensors → ESP32 → ESP32 controls outputs

If the user is VALID

Then ESP32 does three things:

Shows “Access Granted” on OLED

Turns ON green LED and short beep

Sends attendance data to the cloud (Node-RED / MQTT)

If the user is INVALID

Then ESP32:

Shows “Access Denied” on OLED

Turns ON red LED and long beep

Does NOT send attendance as present

Only sends a “Denied” status (for logging / security), but no attendance is recorded

So:

❌ Invalid user → No attendance → Not counted in cloud

In your project, you used an ESP32 development board.

Most common ESP32 boards have:

✅ 38 pins (some have 30 pins, but yours is 38-pin type)

These pins include:

Power pins (3.3V, 5V, GND)

GPIO pins (for LED, buzzer, sensors)

Communication pins (SPI, I2C, UART)

So:

ESP32 = around 38 pins

FINGERPRINT SENSOR → ESP32 (Simple explanation)

The fingerprint sensor needs:

Power

Ground

Two data wires

🔸 Connections

Fingerprint Pin 	Connected to ESP32	Purpose

VCC	              3.3V / 5V	        	Gives power to the sensor

GND	                 GND	       	 	Completes the circuit

TX	                GPIO 16	        	Sends fingerprint data to ESP32

RX	                GPIO 17   		Receives commands from ESP32

🔹 How it works (very easy)

User puts finger on sensor

Sensor reads fingerprint

Sensor sends fingerprint data through TX → ESP32 pin 16

ESP32 sends commands through pin 17 → RX of sensor

ESP32 checks if fingerprint is valid

This communication is called Serial (UART).

🔹 One-line explanation (best for viva)

“The fingerprint sensor is connected to ESP32 using UART pins 16 and 17. Pin 16 receives fingerprint data, pin 17 sends commands, and VCC and GND provide power.”

🔹 RFID READER → ESP32 (Simple explanation)

The RFID reader needs:

Power

Ground

Five data wires

🔸 Connections

RFID Pin	Connected to ESP32		Purpose

VCC			3.3V			Gives power to RFID reader

GND			GND			Completes the circuit

SDA / SS		GPIO 5			Selects RFID reader

RST			GPIO 27			Resets the RFID module

SCK			GPIO 18			Clock signal

MISO			GPIO 19			RFID sends data to ESP32

MOSI			GPIO 23			ESP32 sends data to RFID

What value resistor is used?

In your report:

You used 220Ω(Omega) resistor

This is perfect for:

Green LED

Red LED

1. How many pins (holes) does a breadboard have?

In your project, you used a standard medium breadboard.

Usually:

A normal breadboard has about 400 to 830 holes (pins)

Common types:

Small → 170 holes

Medium → 400 holes ✅ (most common in projects)

Large → 830 holes

So you can safely say:

Breadboard has hundreds of connection holes (around 400–800 holes).

Correct flow of electricity in your project

Electricity always flows like this:

ESP32 power (+) → Power line → Component → Ground line → Back to ESP32 (–)

So:

Power line sends electricity

Component uses electricity

Ground line sends it back to ESP32

This completes the loop.

Power line gets electricity from ESP32

You connect:

ESP32 3.3V pin → Breadboard power line

Now:

The power line becomes full of 3.3V electricity

Any component connected to this line can take electricity

So yes ✅:

🔌 Power line CAN supply electricity

(because ESP32 gives electricity to it)

How does OLED work with ESP32?

ESP32 sends text data to OLED

OLED receives the data

OLED shows the message on the screen

Connections:

VCC → power line (gets power)

GND → ground line

SDA & SCL → ESP32 (data lines)

Communication type:

I2C communication

Fingerprint sensor = 4 pins ✅

RFID reader = 8 pins ✅

Buzzer = 2 pins ✅